附件4

Emission standard of air pollutants for iron and steel industry

Abstract

1 This standard specifies the requirements for the control, monitoring, implementation and supervision of the emission of air pollutants from iron and steel industry enterprises in Shanghai.

2 This standard applies to managing the emission of air pollutants from the existing of iron and steel industry enterprises within Shanghai’s jurisdiction, as well as conducting environmental impact assessments, designing and constructing new, modified or expanded iron and steel industry projects. It also covers completion acceptance of environmental protection measures, discharge permit management, and post-completion air pollutant emission management.

3 For the emission of gas pollutants from iron and steel industry production facilities, the limit requirements specified in this standard shall be implemented before October 1, 2028. the new, modified and expanded iron and steel industry production facilities must implement the standard upon its effective date.

4 The maximum allowable emission limits for flue gas pollutants from iron and steel industry production facilities in this standard are:

units：mg/m3(except for benzo(a)pyrene and dioxins)

| Production process | Production devices | Item | Emission limit values |
| --- | --- | --- | --- |
| Coke production | Coke oven flue | Particulate matter | 10 |
|  |  | Sulfur dioxide (SO2) | 30 |
|  |  | Nitrogen oxides (NOX, as NO2) | 130 |
|  |  | Non-methane hydrocarbons (NMHC) | 100 |
|  |  | Ammoniaa | 8 |
|  | Coal loading | Particulate matter | 10 |
|  |  | Sulfur dioxide (SO2) | 40 |
|  |  | Benzo [a] pyrene (μg/m3) | 0.3 |
|  | Coke pushing | Particulate matter | 10 |
|  |  | Sulfur dioxide (SO2) | 30 |
|  | Coke dry quenching | Particulate matter | 10 |
|  |  | Sulfur dioxide (SO2) | 50 |
|  | Coal crushing, coke crushing, screening, transfer and other facilities requiring ventilation | Particulate matter | 10 |
|  | Tube furnace and other facilities that burn gas | Particulate matter | 10 |
|  |  | Sulfur dioxide (SO2) | 30 |
|  |  | Nitrogen oxides (NOX, as NO2) | 150 |
|  |  | Ammoniaa | 8 |
|  | Ammonium sulfate crystallization and drying | Particulate matter | 10 |
|  |  | Ammonia | 10 |
|  | Cold drum, tar in the reservoir or other storage tank and loading/unloading facilities | Benzo [a] pyrene (μg/m3) | 0.3 |
|  |  | Hydrogen cyanide (HCN) | 1.0 |
|  |  | Phenols | 20 |
|  |  | Non-methane hydrocarbons (NMHC) | 50 |
|  |  | Ammonia | 10 |
|  |  | Hydrogen sulfide (H2S) | 1.0 |
|  | Benzene storage tank and loading/unloading facilities | Benzene | 4 |
|  |  | Non-methane hydrocarbons (NMHC) | 50 |
|  | Desulfurization and acid production unit | Sulfur dioxide (SO2) | 100 |
|  |  | Nitrogen oxides (NOX, as NO2) | 150 |
|  |  | Sulfuric acid mist | 5 |
|  | Desulfurization and regeneration device | Ammonia | 10 |
|  |  | Hydrogen sulfide (H2S) | 1.0 |
|  | Production wastewater treatment facilities | Non-methane hydrocarbons (NMHC) | 50 |
|  |  | Ammonia | 10 |
|  |  | Hydrogen sulfide (H2S) | 1.0 |
| Sinter (pellet) | Sintering machine head, pellet roasting equipment | Particulate matter | 10 |
|  |  | Sulfur dioxide (SO2) | 35 |
|  |  | Nitrogen oxides (NOX, as NO2) | 50 |
|  |  | Fluoride (as F) | 3.0 |
|  |  | Dioxins and furans (ng-TEQ/m3) | 0.2 |
|  |  | Ammoniaa | 8 |
|  | Tail of sintering machine, belt roaster or other devices | Particulate matter | 10 |
| Iron smelting | Hot blast stove | Particulate matter | 10 |
|  |  | Sulfur dioxide (SO2) | 50 |
|  |  | Nitrogen oxides (NOX, as NO2) | 150（100）b |
|  |  | Ammoniaa | 8 |
|  | Blast furnace spout area | Particulate matter | 10 |
|  | Raw material system, powder coal system and other devices | Particulate matter | 10 |
| Steelmaking | Primary flue gas from converter | Particulate matter | 20 |
|  | Muffle furnace, pretreatment of molten iron, Secondary flue gas from converter, electric furnace, refining furnace |  | 10 |
|  | Continuous casting cutting, flame descaling and other devices |  | 10 |
|  | Steel slag treatment |  | 20 |
|  | Electric furnace | Dioxins and furans (ng-TEQ/m3) | 0.2 |
|  | Electroslag metallurgy | Fluoride (as F) | 5.0 |
| Steel rolling | Heat treatment furnace | Particulate matter | 10 |
|  |  | Sulfur dioxide (SO2) | 50 |
|  |  | Nitrogen oxides (NOX, as NO2) | 150 |
|  | Hot finishing mill, straightening, finishing, shot blasting, polishing, welding machine and other devices | Particulate matter | 10 |
|  | Rolling mill | Oil mist | 20 |
|  | Pickling line | Hydrogen chloride (HCl) | 15 |
|  |  | Sulfuric acid mist | 10 |
|  |  | Chromate mist | 0.06 |
|  |  | Nitric acid mist (as NO2) | 150 |
|  |  | Fluoride (as F) | 6.0 |
|  | Acid waste regeneration | Particulate matter | 10 |
|  |  | Hydrogen chloride (HCl) | 30 |
|  |  | Nitric acid mist (as NO2) | 150 |
|  |  | Fluoride (as F) | 6.0 |
|  |  | Sulfur dioxide (SO2) | 100 |
|  |  | Nitrogen oxides (NOX, as NO2) | 200 |
|  | Skim | Alkali mist | 10 |
|  | Coating unit | Benzene | 0.5 |
|  |  | Toluene | 5.0 |
|  |  | Xylene | 10 |
|  |  | Non-methane hydrocarbons (NMHC) | 50 |
|  |  | Chromate mistc | 0.06 |
| Rotary hearth furnace | Pre-treatment dryer | Particulate matter | 10 |
|  |  | Sulfur dioxide (SO2) | 50 |
|  |  | Nitrogen oxides (NOX, as NO2) | 150 |
|  | Chimney of rotary hearth furnace | Particulate matter | 10 |
|  |  | Sulfur dioxide (SO2) | 50 |
|  |  | Nitrogen oxides (NOX, as NO2) | 150 |
|  |  | Zinc and its compounds (as Zn) | 10 |
|  |  | Lead and its compounds (as Pb) | 0.1 |
|  | Material storage, transfer, and other production facilities | Particulate matter | 10 |
| a Apply to facilities for ammonia desulfurization and denitrification. b For new blast furnace construction projects (including reconstruction and expansion), the nitrogen oxide emissions in the flue gas from the corresponding hot blast stoves shall be subject to a limit of 100 mg/m³. c Apply to chromium passivation process. |  |  |  |

The exhaust gas from coke oven flue, sintering machine head, pellet shaft furnace, grate-kiln, belt type pellet roasting machine head, heat treatment furnace (including reheating furnace) and rotary hearth furnace are standardized at a reference oxygen content listed in the table below in waste gas.

| Number | Production devices | Reference oxygen content，% |
| --- | --- | --- |
| 1 | Coke oven flue | 8 |
| 2 | Sintering machine head and pellet shaft furnace | 16 |
| 3 | Grate-kiln and belt type pellet roasting machine head | 18 |
| 4 | Reheating furnace | 8 |
| 5 | Other heat treatment furnace | 15 |
| 6 | Acid waste regeneration | 9 |
| 7 | Chimney of rotary hearth furnace | 16 |

VOCs combustion (incineration/oxidation) units shall, in addition to meeting the atmospheric pollutant emission requirements specified in Table 1, also control sulfur dioxide and nitrogen oxides in the exhaust gas to achieve the limits set forth in Table below.

| Number | Item | Emission limit values |
| --- | --- | --- |
| 1 | Sulfur dioxide (SO2) | 100 |
| 2 | Nitrogen oxides (NOX, as NO2) | 150 |

Where supplementary air is required for combustion or oxidation reactions in VOCs combustion (incineration/oxidation) units, the measured atmospheric pollutant emission concentration in the exhaust stack shall be converted to the reference atmospheric pollutant emission concentration at a reference oxygen content of 3%. For organic waste gas treated by incineration in boilers, coke ovens, other industrial furnaces, or solid waste incinerators, the reference oxygen content of the flue gas shall be implemented in accordance with the provisions of their respective emission standards. Where the oxygen content of the waste gas entering the VOCs combustion (incineration/oxidation) unit is sufficient to meet the needs of its own combustion or oxidation reactions without additional supplementary air, or where supplementary air is required only for the burner, the measured mass concentration shall serve as the basis for compliance determination; however, the oxygen content of the flue gas at the unit outlet shall not exceed that of the waste gas at the unit inlet. For other VOCs treatment facilities, including adsorption, absorption, condensation, biological treatment, and membrane separation, the measured mass concentration shall serve as the basis for compliance determination, and dilution discharge is prohibited.

5 The maximum allowable emission limits for unorganized emission of gas pollutants from iron and steel industry production facilities in this standard are:

| Production process | Production devices | Item | Emission limit values |
| --- | --- | --- | --- |
| Sinter (pellet), iron smelting, steelmaking | Workshop with plant | Particulate matter (TSP) | 8.0 |
|  | Workshop without complete plant |  | 5.0 |
| Steel rolling | Slab heating,、grinding roller, steel coil finishing, acid regeneration feeding | Particulate matter (TSP) | 5.0 |
|  | Pickling line, waste acid regeneration | Sulfuric acid mist | 1.2 |
|  |  | Hydrogen chloride (HCl) | 0.2 |
|  |  | Nitric acid mist (as NO2) | 0.12 |
|  | Coating line | Benzene | 0.1 |
|  |  | Toluene | 0.6 |
|  |  | Xylene | 0.2 |
|  |  | Non-methane hydrocarbons (NMHC) | 2.0 |
| Coke production | Top of coke oven | Particulate matter (TSP) | 2.5 |
|  |  | Benzo [a] pyrene (μg/m3) | 2.5 |
|  |  | Hydrogen sulfide (H2S) | 0.1 |
|  |  | Ammonia | 2.0 |
|  |  | Benzene soluble particulate matter | 0.6 |
|  | Within the confines of the factory | Particulate matter (TSP) | 2.5 |
|  |  | Non-methane hydrocarbons (NMHC) | 6 (1-hour average concentration value at the monitoring point) 20 (any concentration value at the monitoring point)a |
| Enterprise boundary | All | Particulate matter (TSP) | 1.0 |
|  |  | Ammonia | 0.2 |
|  | Involving coking process | Benzo [a] pyrene (μg/m3) | 0.01 |
|  |  | Hydrogen cyanide (HCN) | 0.02 |
|  |  | Benzene | 0.1 |
|  |  | Phenols | 0.02 |
|  |  | Hydrogen sulfide (H2S) | 0.01 |
|  |  | Non-methane hydrocarbons (NMHC) | 2.0 |
| a The monitoring of any concentration value of NMHC in the factory area shall be carried out in accordance with the relevant monitoring technology regulations of portable monitoring instruments. |  |  |  |